For SystemC LT (Loosely Timed) modeling, SIUL/SIUL2 act as pinmux/GPIO/interrupt controllers, while CAN and CAN-FD are communication IPs modeled at transaction level. The crisp takeaway: SIUL/SIUL2 are modeled as register-level configuration blocks, whereas CAN/CAN-FD are modeled as protocol-level TLM sockets with latency and buffering.

🔧 SIUL / SIUL2 in SystemC LT

- Role: System Integration Unit Lite (SIUL/SIUL2) configures MCU pins, multiplexing, GPIO, and external interrupts.

- Modeling Approach:

  - Registers as TLM targets: MSCR (pad config), IMCR (input mux), GPIO data registers.

  - Interrupt signaling: Modeled as events/notifications in SystemC.

  - Abstraction: LT modeling uses functional calls (write(addr, data)) with latency annotations, not cycle-accurate toggling.

- Use Case in LT: Early software validation (pinmux setup, GPIO toggling) without RTL detail.

🚍 CAN vs. CAN-FD in SystemC LT

- CAN (Classical):

  - Frame size: Up to 8 bytes payload.

  - LT Modeling: Transaction = frame send/receive, modeled as TLM sockets.

  - Latency: Approximate arbitration + transmission delay modeled as sc_time.

  - Error handling: Basic error state transitions (error active/passive).

- CAN-FD (Flexible Data Rate):

  - Frame size: Up to 64 bytes payload, faster data phase.

  - LT Modeling Enhancements:

    - Extended payload handling in transaction objects.

    - Dual-phase timing (arbitration vs. data phase).

    - Backward compatibility: CAN-FD model must accept classical CAN frames.

  - Validation: Often tested cross-platform with tools like Vector CANoe + SilKit, enabling virtual ECU-level verification before RTL leadsoc.com.

📊 Comparison Table

| Aspect | SIUL/SIUL2 LT Model | CAN LT Model | CAN-FD LT Model |
| --- | --- | --- | --- |
| Function | Pinmux, GPIO, interrupts | Classical CAN bus frames | Extended CAN bus frames |
| Interface | Register TLM target | TLM initiator/target sockets | Same, with extended payload |
| Latency Model | Register access delay | Arbitration + frame delay | Arbitration + dual-phase delay |
| Payload | N/A | 0–8 bytes | 0–64 bytes |
| Error Handling | Interrupt events | Error states (active/passive) | Error states + FD-specific |
| Use Case | Early SW pin config | ECU comm validation | High-bandwidth ECU comm validation |

⚠️ Practical Notes

- SIUL/SIUL2: Keep models lightweight—focus on register map + interrupt events. Avoid cycle-accurate pad toggling in LT.

- CAN/CAN-FD: LT models should abstract bus arbitration but allow latency injection for realistic timing.

- Integration: In a virtual ECU, SIUL configures pins → CAN controller uses those pins → CAN/CAN-FD TLM sockets simulate bus traffic.

- Testing: CAN-FD LT models are validated against real-world tools (CANoe, SilKit) to ensure protocol correctness leadsoc.com.

✅ Crisp takeaway for SystemC LT modeling:

- SIUL/SIUL2 → Register-level pinmux/GPIO/interrupt abstraction.

- CAN → Transaction-level frame exchange (≤8 bytes).

- CAN-FD → Extended transaction model (≤64 bytes, dual-phase timing).